Note on field testing and data processing of CPT

Yan-Guo Zhou*, Yue Wang

MOE Key Laboratory of Soft Soils and Geoenvironmental Engineering, Institute of Geotechnical Engineering, Center for Hypergravity Experimental and Interdisciplinary Research, Zhejiang University, Hangzhou 310058, P. R. China. *E-mail: qzking@zju.edu.cn

March 25, 2019

According to previous study, we recommend the following definition of related parameters.

1. TE, Time of earthquake occurrence.

2. σvo is the total vertical overburden stress; σvo' is the effective vertical overburden stress.

3.amax, the peak acceleration of ground.

4.rd is the nonlinear shear mass participation factor ,is nonlinearly dependent upon a suite of factors that include soil depth, average Vs of the soil ,the amplitude of ground motion, and earthquake magnitude.

5. CSR, cyclic stress ratio.

6. MSF, magnitude scaling factor.

7. DWF, Earthquake Duration Weighting Factor.

8. Rf, friction ratio the ratio of sleeve to tip resistance, fs /qc, in percent.

9. qc=cone tip resistance, fs =cone sleeve resistance ;

qc,1=normalized cone tip resistance ,

fs,1 =normalized cone sleeveresistance.

10. c and s =tip and sleeve normalization exponents.

11. Cq and Cf =tip and sleeve normalization factor of CPT.

12.GC-gravel content；CC-clay content ；FC-fine content.

13.PI-plastic index

14. d50-characteristic particle size

15. Kσ is overburden correction factor.

16. Ic, a soil behavior type index which correlating with the FC and soil classification.

17. Q and F are normalized tip and sleeve friction ratios.

Field testing and data processing

- Data Quality Assessment

A rating system was established to evaluate the quality of each field case history.

Data were rated as falling into one of five classes (from highest to lowest quality) as follows:

Class A

a. Original CPT trace with qc and fs/Rf, using an ASTM D3441 and D5778 spec. cone.

b. COV（CSR）≤0.20

Class B

a. Original CPT trace with qc and fs/Rf, using an ASTM D3441 and D5778 spec. cone.

b. 0.2＜COV（CSR）≤0.35

Class C

a. Original CPT trace with qc and fs/Rf, but using a nonstandard cone e.g., Chinese cone or mechanical cone.

b.No sleeve data but fines content FC≤5% （i.e., “clean” sand）.

c. 0.35＜COV（CSR）≤0.5.

Class D

a. Not satisfying the criteria for Classes A, B, or C.

- Selection and calculation of parameters

Criteria of selection and calculation refers to previous study (e.g., Moss, Cetin，Kayen, Dobry.R).

1)  Magnitude Mw is used for the magnitude, other magnitudes such as Richter scale, near-magnitude ML, surface wave magnitude Ms, body wave magnitude Mb, etc. need to be converted into moment magnitude, and indicate conversion, note the original data. The formula for MSF (magnitude scaling factor) is as follows

(1)

where MSFmax was related to qc1N values as

(2)

2) σvo is the total vertical overburden stress; σvo' is the effective vertical overburden stress.

3) rd is the nonlinear shear mass participation factor, is nonlinearly dependent upon a suite of factors that include soil depth, average Vs of the soil, the amplitude of ground motion, and earthquake magnitude. The mean rd results can be calculated using the following equation:

for d＜20 m

(3)

and for d≥20m

(4)

where d=depth in meters at the midpoint of the critical layer; MW=moment magnitude; and amax=peak ground acceleration in units of gravity. The standard deviation for rd is as follows:

for d＜12.2 m

(5)

and for d≥12.2 m is

(6)

4) CSR is cyclic stress ratio，and CSR is a measure of seismic demand on a soil element and has been represented in the simplified method first proposed by Seed and Idriss (1971, 1982) as

（7）

where amax=peak acceleration of ground in units of gravity; σ vo is total vertical overburden stress; σ vo' is effective vertical overburden stress; rd is the nonlinear shear mass participation factor; K= correction for overburden stress.

5) Idriss and Boulanger (2008) recommended that the resulting K relationship as follows:

(8)

where Pa= normalized reference stress (1 atm=101.352kPa).

6) The method for determining critical layers

Selection of the critical layer is an important step in estimating the seismic strength of a particular soil deposit. The criteria for selection is finding the strata of soil that is the weakest-link-in-the-chain from a liquefaction perspective. Finding the weakest link requires observing the tip resistance and friction ratio in conjunction, with the addition of a SPT log, for soil classification, if one is available. For most depositional environments this can be a simple matter of looking for the smallest continuous stretch of tip resistance with low friction ratio that agrees with the SPT log in terms of a liquefiable material. For most depositional environments this can be a simple matter of looking for the smallest continuous stretch of tip resistance with low friction ratio that agrees with the boring log in terms of the material texture （i.e., cohesive versus cohesionless soils).

This can be a difficult task for fluvial deposits, which tend to be thin, interbedded, and discontinuous both horizontally and vertically. A second method for identifying a critical layer is to calculate the profile of cyclic resistance ratio CRR based on existing correlations and take the critical layer as the domain with the smallest CRR. This method is used for the case histories where multiple candidates for the critical layer exist and it is unclear which of these layers liquefied or did not liquefy.

7) qc,1 is normalized cone tip resistance in CPT test; fs,1 is normalized cone sleeve resistance in CPT test.

Cq and Cf is used to normalize the tip and sleeve resistance for a given level of overburden stress, using the following equations:

（9）

where

（10）

where qc,1=normalized tip resistance (MPa）; Cq=tip normalization factor; qc =raw tip resistance（MPa）; Pa=reference stress in compatible units (1 atm=101.352kPa); σv’=effective overburden stress (same units as Pa); c=normalization exponent. Cq and Cf should not exceed a value of 1.7.

（11）

where

x1=0.78,x2=-0.33,y1=-0.32,y2=-0.35,y3=0.49,z1=1.21.c=s and Cq=Cf.

8) CPT penetration resistances also can be corrected for overburden stress effects as

（12）

where CN = overburden correction factor; Pa = atmospheric pressure(1 atm=101.352 kPa); qcN =qc=Pa; and qc1N = penetration resistance that would be obtained in the same sand at an overburden stress of 1 atm if all other attributes remain constant (e.g., same relative density, fabric, age, degree of cementation, and loading history). The variable qc should be corrected for pore pressures measured behind the tip whenever such data are available. The CN relationship by Boulanger (2003) based on calibration chamber test data and numerical modeling of cone penetration was used

（13）

where qc1Ncs = equivalent clean sand penetration resistance (as discussed below). The exponent m can be constrained to its recommend limits of 0.264 ≤ m ≤ 0.782 by limiting qc1Ncs values to between 21 and 254 for use in the above expression.

9) The Ic term by Robertson and Wride (1998) is

(14)

where Q and F are normalized tip and sleeve friction ratios computed as

(15)

(16)

The exponent n varies from 0.5 in sands to 1.0 in clays (Robertson and Wride 1998).

- Standard of soil classification

Adopting USCS(Unified Soil Classification System) standard. （https://scholar.google.com.hk）

Refernces:

[1] Kayen, R., Moss, R. E. S., Thompson, E. M., Seed, R. B., Cetin, K. O., & Kiureghian, A. D., et al. (2013). Shear-Wave Velocity–Based Probabilistic and Deterministic Assessment of Seismic Soil Liquefaction Potential. Journal of Geotechnical and Geoenvironmental Engineering.

[2] Kayen, R., Thompson, E., Minasian, D., Moss, R. E., Collins, B. D., Sitar, N., ... & Carver, G. (2004). Geotechnical reconnaissance of the 2002 Denali fault, Alaska, earthquake. Earthquake Spectra, 20(3), 639-667.

[3] Cetin, K. O., Seed, R. B., Der Kiureghian, A., Tokimatsu, K., Harder Jr, L. F., Kayen, R. E., & Moss, R. E. (2004). Standard penetration test-based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of geotechnical and geoenvironmental engineering, 130(12), 1314-1340.

[4] Moss, R. E. (2003). CPT-Based Probabilistic Assessment of Seismic Soil Liquefaction Initiation. Dissertation.

[5] Boulanger, R. W., & Idriss, I. M. (2014). CPT and SPT based liquefaction triggering procedures Report No. UCD/CGM-14/01 Center for Geotechnical Modeling (Davis USA: University of California, Davis) 134 Department of Civil and Environmental Engineering Report.

[6] Moss, R. E., Seed, R. B., Kayen, R. E., Stewart, J. P., Der Kiureghian, A., & Cetin, K. O. (2006). CPT-based probabilistic and deterministic assessment of in situ seismic soil liquefaction potential. Journal of Geotechnical and Geoenvironmental Engineering, 132(8), 1032-1051.

[7] Cetin, K. O., Seed, R. B., Der Kiureghian, A., Tokimatsu, K., Harder Jr, L. F., Kayen, R. E., & Moss, R. E. (2004). Standard penetration test-based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of geotechnical and geoenvironmental engineering, 130(12), 1314-1340.

[8] Boulanger, R. W., & Idriss, I. M. (2015). CPT-based liquefaction triggering procedure. Journal of Geotechnical and Geoenvironmental Engineering, 142(2), 04015065.