Note on field testing and data processing of SPT

Yan-Guo Zhou*, Yue Wang

MOE Key Laboratory of Soft Soils and Geoenvironmental Engineering, Institute of Geotechnical Engineering, Center for Hypergravity Experimental and Interdisciplinary Research, Zhejiang University, Hangzhou 310058, P. R. China. *E-mail: qzking@zju.edu.cn

March 25, 2019

According to previous study, we recommend the following definition of related parameters.

1. TE, Time of earthquake occurrence.

2.σvo is the total vertical overburden stress; σvo' is the effective vertical overburden stress.

4. amax, the peak acceleration of ground.

5. rd is the nonlinear shear mass participation factor ,is nonlinearly dependent upon a suite of factors that include soil depth, average Vs of the soil ,the amplitude of ground motion, and earthquake magnitude.

6. CSR, cyclic stress ratio.

7. MSF, magnitude scaling factor.

8. DWF, Earthquake Duration Weighting Factor.

9. CR, CS, CB, CE and CV are the correction of N1. where CR=correction for rod length; CS=correction for nonstandardized sampler configuration; CB=correction for borehole diameter; and CE=correction for hammer energy efficiency.

10. GC-gravel content; CC-clay content; FC-fine content.

11. PI-plastic index

12. d50 -characteristic particle size

13. Kσ is overburden correction factor.

Field testing and data processing

- Data Quality Assessment

A rating system was established to evaluate the quality of each field case history.

Data were rated as falling into one of five classes (from highest to lowest quality) as follows:

Class A

a. A minimum of 3 or more N values in the critical stratum,

b. Equipment and procedural details affecting SPT data well-defined, and

c. COV（CSR）≤ 0.20.

Class B

a. Equipment and procedural details affecting SPT data well-defined, and

b. 0.2＜COV（CSR）≤0.35, or satisfies Class A but less than 3 N values in the critical stratum.

Class C

a. Equipment and procedural details affecting SPT data well-defined, and

b. 0.35＜COV（CSR）≤0.5.

Class D

a. Equipment and procedural details affecting SPT data not well defined, and

b. Seismicity, and/or site effects not well-defined [COV（CSR）＞0.5], but some reasonable basis for at least approximate estimation of CSR available,

c. Poor site performance data/documentation, or

d. Original boring logs or other important data not directly accessible, etc.

- Selection and calculation of parameters

Criteria of selection and calculation refers to previous study (e.g., Moss, Cetin，Kayen, Dobry.R).

1) Magnitude Mw is used for the magnitude, other magnitudes such as Richter scale, near-magnitude ML, surface wave magnitude Ms, body wave magnitude Mb, etc. need to be converted into moment magnitude, and indicate conversion, note the original data. MSF is magnitude scaling factor, which for a given earthquake moment magnitude, Mw, is given by the following expression (Boulanger and Idriss 2008):

(1)

2) σvo is the total vertical overburden stress; σvo' is the effective vertical overburden stress.

3) rd is the nonlinear shear mass participation factor, is nonlinearly dependent upon a suite of factors that include soil depth, average Vs of the soil, the amplitude of ground motion, and earthquake magnitude. The mean rd results can be calculated using following equation

for d＜20 m

(2)

and for d≥20 m

(3)

where d= depth in meters, measured at the midpoint of the critical layer; and amax = peak ground acceleration in units of gravity.

The standard deviation for rd for d＞12.2 m is

And for d≤12.2 m is

4) CSR is cyclic stress ratio，and CSR is a measure of seismic demand on a soil element and has been represented in the simplified method first proposed by Seed and Idriss (1971, 1982) as Eq.(3)

（4）

where amax=peak acceleration of ground in units of gravity; σvo is total vertical overburden stress; σ vo' is effective vertical overburden stress; rd is the nonlinear shear mass participation factor; K= correction for overburden stress.

5) Idriss and Boulanger (2008) recommended that the resulting K relationship as follows:

(5)

where Pa= normalized reference stress (1 atm=101.352kPa).

6) The method for determining critical layers

Selection of the critical layer is an important step in estimating the seismic strength of a particular soil deposit. The criteria for selection is finding the strata of soil that is the weakest-link-in-the-chain from a liquefaction perspective. Finding the weakest link requires observing the tip resistance and friction ratio in conjunction, with the addition of a SPT log, for soil classification, if one is available. For most depositional environments this can be a simple matter of looking for the smallest continuous stretch of tip resistance with low friction ratio that agrees with the SPT log in terms of a liquefiable material. For most depositional environments this can be a simple matter of looking for the smallest continuous stretch of tip resistance with low friction ratio that agrees with the boring log in terms of the material texture （i.e., cohesive versus cohesionless soils).

This can be a difficult task for fluvial deposits, which tend to be thin, interbedded, and discontinuous both horizontally and vertically. A second method for identifying a critical layer is to calculate the profile of cyclic resistance ratio CRR based on existing correlations and take the critical layer as the domain with the smallest CRR. This method is used for the case histories where multiple candidates for the critical layer exist and it is unclear which of these layers liquefied or did not liquefy.

7）The resulting N1 values were then further corrected for energy, equipment, and procedural effects to fully standardized N1,60 values as Eq.（6）

（6）

where CR=correction for “short” rod length; CS=correction for nonstandardized sampler configuration; CB=correction for borehole diameter; and CE=correction for hammer energy efficiency.

The corrections for CR CS CB CE are summarized as follows:

CR   See following figure for Rod Length Correction Factors.

CS     For samplers with an indented space for interior liners, but with liners omitted during sampling

With limits as 1.10≤CS≤1.30

CB    When Borehole diameter ranges from 65 to 115 mm，CB=1.00；

when Borehole diameter is 150 mm，CB=1.05；

when Borehole diameter is 200 mm，CB=1.15；

CE

where ER (efficiency ratio) is the fraction or percentage of the theoretical SPT impact hammer energy actually transmitted  to the sampler, expressed as %.

ER can be estimated。For good field procedures, equipment and monitoring, the following guidelines are suggested:

| Equipment | Approximate ER |
| --- | --- |
| Safety Hammer | 0.4 to 0.75 |
| Donut Hammer 1 | 0.3 to 0.6 |
| Donut Hammer 2 | 0.7 to 0.85 |
| Automatic-Trip Hammer | 0.5 to 0.8 |

* Safety Hammer and Donut Hammer 1：Based on rope and cathead system, two turns of rope around cathead, “normal” release (not the Japanese “throw”), and rope not wet or excessively worn.

* Donut Hammer 2：Rope and cathead with special Japanese “throw” release.

- Standard of soil classification

Adopting USCS(Unified Soil Classification System) standard.

（https://scholar.google.com.hk）

Refernces:

[1] Kayen, R., Moss, R. E. S., Thompson, E. M., Seed, R. B., Cetin, K. O., & Kiureghian, A. D., et al. (2013). Shear-Wave Velocity–Based Probabilistic and Deterministic Assessment of Seismic Soil Liquefaction Potential. Journal of Geotechnical and Geoenvironmental Engineering.

[2] Kayen, R., Thompson, E., Minasian, D., Moss, R. E., Collins, B. D., Sitar, N., ... & Carver, G. (2004). Geotechnical reconnaissance of the 2002 Denali fault, Alaska, earthquake. Earthquake Spectra, 20(3), 639-667.

[3] Cetin, K. O., Seed, R. B., Der Kiureghian, A., Tokimatsu, K., Harder Jr, L. F., Kayen, R. E., & Moss, R. E. (2004). Standard penetration test-based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of geotechnical and geoenvironmental engineering, 130(12), 1314-1340.

[4] Moss, R. E. (2003). CPT-Based Probabilistic Assessment of Seismic Soil Liquefaction Initiation. Dissertation.

[5] Boulanger, R. W., & Idriss, I. M. (2014). CPT and SPT based liquefaction triggering procedures Report No. UCD/CGM-14/01 Center for Geotechnical Modeling (Davis USA: University of California, Davis) 134 Department of Civil and Environmental Engineering Report.

[6] Moss, R. E., Seed, R. B., Kayen, R. E., Stewart, J. P., Der Kiureghian, A., & Cetin, K. O. (2006). CPT-based probabilistic and deterministic assessment of in situ seismic soil liquefaction potential. Journal of Geotechnical and Geoenvironmental Engineering, 132(8), 1032-1051.

[7] Cetin, K. O., Seed, R. B., Der Kiureghian, A., Tokimatsu, K., Harder Jr, L. F., Kayen, R. E., & Moss, R. E. (2004). Standard penetration test-based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of geotechnical and geoenvironmental engineering, 130(12), 1314-1340.

[8] Boulanger, R. W., & Idriss, I. M. (2015). CPT-based liquefaction triggering procedure. Journal of Geotechnical and Geoenvironmental Engineering, 142(2), 04015065.