Note on field testing and data processing of Vs

Yan-Guo Zhou*, Yue Wang

MOE Key Laboratory of Soft Soils and Geoenvironmental Engineering, Institute of Geotechnical Engineering, Center for Hypergravity Experimental and Interdisciplinary Research, Zhejiang University, Hangzhou 310058, P. R. China. *E-mail: qzking@zju.edu.cn

March 25, 2019

According to previous study, we recommend the following definition of related parameters.

1. TE, Time of earthquake occurrence.

2. σvo is the total vertical overburden stress; σvo' is the effective vertical overburden stress.

4. amax, the peak acceleration of ground.

5. rd is the nonlinear shear mass participation factor ,is nonlinearly dependent upon a suite of factors that include soil depth, average Vs of the soil ,the amplitude of ground motion, and earthquake magnitude.

6. CSR, cyclic stress ratio.

7. MSF, magnitude scaling factor.

8. DWF, earthquake duration weighting factor.

9. Cvs , stress correction, is affected by the vertical effective overburden stress (σ vo'), normalized reference stress (Pa).

10. GC-gravel content；CC-clay content；FC-fine content.

11. PI-plastic index

12. d50 -characteristic particle size

15. Kσ is overburden correction factor.

Field testing and data processing

- Data Quality Assessment

A rating system was established to evaluate the quality of each field case history. Data Class criteria refers to previous study.

In the Vs sheet, the case histories were evaluated for seismic demand (CSR)，and then classified according to the quality of the data. Four classes of data, A through D, were used to group the case histories, with D being substandard and excluded from the final case history database. The criteria for the data classes depends on the standard deviation of the CSR value (COV（CSR)）within the critical layer.

Class A

a. Original CPT trace with qc and fs/Rf, using an ASTM D3441 and D5778 spec. cone.

b. COV（CSR）≤0.20.

Class B

a. Original CPT trace with qc and fs/Rf, using an ASTM D3441 and D5778 spec. cone.

b. 0.2＜COV（CSR）≤0.35.

Class C

a. Original CPT trace with qc and fs/Rf, but using a nonstandard cone (e.g., Chinese cone or mechanical cone).

b. No sleeve data but fines content FC 5% (i.e.,“clean” sand).

c. 0.35＜COV（CSR）≤0.5.

Class D

a. Not satisfying the criteria for Classes A, B, or C.

- Selection and calculation of parameters

Criteria of selection and calculation refers to previous study (e.g., Moss, Cetin，Kayen, Dobry.R).

1) Magnitude Mw is used for the magnitude, other magnitudes such as Richter scale, near-magnitude ML, surface wave magnitude Ms, body wave magnitude Mb, etc. need to be converted into moment magnitude, and indicate conversion, note the original data. MSF is magnitude scaling factor, which for a given earthquake moment magnitude, Mw, is given by the following expression (Boulanger and Idriss 2008):

(1)

2) σ vo is the total vertical overburden stress; σ vo' is the effective vertical overburden stress.

3) rd is the nonlinear shear mass participation factor, is nonlinearly dependent upon a suite of factors that include soil depth, average Vs of the soil, the amplitude of ground motion, and earthquake magnitude. The mean rd results can be calculated using following equation

for d＜20 m

(2)

where d= depth in meters, measured at the midpoint of the critical layer; V*s,12m = average Vs in the upper 12.2 m (40 ft) of the soil column; and amax = peak ground acceleration in units of gravity.

The standard deviation for rd for d＜12.2 m is

and for d≥12.2 m is

4) CSR is cyclic stress ratio，and CSR is a measure of seismic demand on a soil element and has been represented in the simplified method first proposed by Seed and Idriss (1971, 1982) as Eq.(3)

（3）

where amax=peak acceleration of ground in units of gravity; σvo is total vertical overburden stress; σ vo' is effective vertical overburden stress; rd is the nonlinear shear mass participation factor; K= correction for overburden stress.

5) Cetin (2004) recommended that the resulting K relationship as follows:

(4)

where Pa= normalized reference stress (1 atm=101.352 kPa)

6) The method for determining critical layers

Selection of the critical layer is an important step in estimating the seismic strength of a particular soil deposit. The criteria for selection is finding the strata of soil that is the weakest-link-in-the-chain from a liquefaction perspective. Finding the weakest link requires observing the tip resistance and friction ratio in conjunction, with the addition of a SPT log, for soil classification, if one is available. For most depositional environments this can be a simple matter of looking for the smallest continuous stretch of tip resistance with low friction ratio that agrees with the SPT log in terms of a liquefiable material. For most depositional environments this can be a simple matter of looking for the smallest continuous stretch of tip resistance with low friction ratio that agrees with the boring log in terms of the material texture （i.e., cohesive versus cohesionless soils).

This can be a difficult task for fluvial deposits, which tend to be thin, interbedded, and discontinuous both horizontally and vertically. A second method for identifying a critical layer is to calculate the profile of cyclic resistance ratio CRR based on existing correlations and take the critical layer as the domain with the smallest CRR. This method is used for the case histories where multiple candidates for the critical layer exist and it is unclear which of these layers liquefied or did not liquefy.

7) The value of Vs1 can be calculated by using the following Eq.（5）

（5）

where stress correction C vs is affected by the vertical effective overburden stress (σ’v), Pa= normalized reference stress (1 atm= 101.352 kPa) and the stress exponent 0.25.

- Standard of soil classification

Adopting USCS(Unified Soil Classification System) standard.

（https://scholar.google.com.hk）

Refernces:

[1] Kayen, R., Moss, R. E. S., Thompson, E. M., Seed, R. B., Cetin, K. O., & Kiureghian, A. D., et al. (2013). Shear-Wave Velocity–Based Probabilistic and Deterministic Assessment of Seismic Soil Liquefaction Potential. Journal of Geotechnical and Geoenvironmental Engineering.

[2] Kayen, R., Thompson, E., Minasian, D., Moss, R. E., Collins, B. D., Sitar, N., ... & Carver, G. (2004). Geotechnical reconnaissance of the 2002 Denali fault, Alaska, earthquake. Earthquake Spectra, 20(3), 639-667.

[3] Cetin, K. O., Seed, R. B., Der Kiureghian, A., Tokimatsu, K., Harder Jr, L. F., Kayen, R. E., & Moss, R. E. (2004). Standard penetration test-based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of geotechnical and geoenvironmental engineering, 130(12), 1314-1340.

[4] Moss, R. E. (2003). CPT-Based Probabilistic Assessment of Seismic Soil Liquefaction Initiation. Dissertation.

[5] Boulanger, R. W., & Idriss, I. M. (2014). CPT and SPT based liquefaction triggering procedures Report No. UCD/CGM-14/01 Center for Geotechnical Modeling (Davis USA: University of California, Davis) 134 Department of Civil and Environmental Engineering Report.

[6] Moss, R. E., Seed, R. B., Kayen, R. E., Stewart, J. P., Der Kiureghian, A., & Cetin, K. O. (2006). CPT-based probabilistic and deterministic assessment of in situ seismic soil liquefaction potential. Journal of Geotechnical and Geoenvironmental Engineering, 132(8), 1032-1051.

[7] Cetin, K. O., Seed, R. B., Der Kiureghian, A., Tokimatsu, K., Harder Jr, L. F., Kayen, R. E., & Moss, R. E. (2004). Standard penetration test-based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of geotechnical and geoenvironmental engineering, 130(12), 1314-1340.

[8] Boulanger, R. W., & Idriss, I. M. (2015). CPT-based liquefaction triggering procedure. Journal of Geotechnical and Geoenvironmental Engineering, 142(2), 04015065.